ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Disc Brakes Squealing or Rubbing

Brakes     Document TS-002     Rev. A

| Applies to | Trailblazer MTB, Summit Trail, Velocity Road (disc), PowerRide e-Bike |
| --- | --- |
| Difficulty | Beginner – Intermediate |
| Time required | 15 – 30 minutes |
| Tools needed | 5 mm hex key, Torque wrench, Isopropyl alcohol & clean rag, Rotor truing fork (optional) |

## Symptoms

- A high-pitched squeal or honk when braking.

- A steady tick or 'shhh' once per wheel revolution while coasting.

- Braking power feels normal but is accompanied by noise.

- Noise is worse in wet conditions or right after washing the bike.

Figure 1. Disc-brake caliper straddles the rotor; pad-to-rotor clearance is only a fraction of a millimetre.

## Likely Causes

- Caliper is slightly off-center so a pad lightly drags the rotor.

- Rotor has minor lateral runout (a small bend or warp).

- Pad or rotor contamination from spray lubricant, degreaser, or oily fingers.

- Brand-new pads/rotor not yet bedded in.

- Loose caliper mount or rotor bolts.

Safety first - A rotor gets very hot during braking — let it cool before touching. - Never touch the braking surface with bare fingers; skin oil contaminates pads. - Avoid spray lubricants anywhere near the rotor or caliper.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Spin the wheel and look through the caliper slot to see which pad is rubbing the rotor.

- Loosen the two caliper mount bolts about a quarter turn so the caliper can float.

- Squeeze and hold the brake lever firmly; while holding, re-tighten the two mount bolts evenly. This self-centers the caliper.

- Release the lever and spin the wheel — the rub should be gone or greatly reduced.

- If a squeal persists, clean the rotor with isopropyl alcohol and a clean rag, then dry it.

- Bed in the pads: ride at moderate speed and brake firmly 20–30 times to deposit an even pad layer.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Check rotor runout with a truing fork or zip-tie pointer; correct bends >0.2 mm by gently flexing the high spot with a rotor truing tool.

- Confirm rotor bolts (6-bolt) are torqued in a star pattern; check Center-Lock lockring torque.

- Inspect pads for glazing or contamination; deglaze lightly with sandpaper on a flat surface, or replace if oil-soaked.

- Verify the wheel is fully seated in the dropouts and the axle/quick-release is at correct torque — a poorly seated wheel mimics rotor rub.

- For persistent resonant squeal, apply a thin film of disc-brake-rated anti-squeal paste to the back of the pads (never the friction face).

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Caliper mount bolts | 6 – 8 N·m |
| 6-bolt rotor bolts | 4 – 6 N·m (star pattern) |
| Center-Lock lockring | 40 N·m |
| Max rotor lateral runout | 0.2 mm |
| Thru-axle | 12 – 15 N·m |

Figure 2. Re-center the caliper by holding the lever while tightening the mount bolts.

Pro tips - Always wash the bike with the rotors pointed away from spray, and keep degreaser well clear. - Handle rotors by the spider arms, not the braking track.

## When to Visit a Service Center

- Rotor runout cannot be corrected or the rotor is cracked.

- Pads are oil-contaminated and squeal returns after replacement.

- Noise is accompanied by a loss of braking power.

- You suspect a seized caliper piston (one pad never retracts).

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.