ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Gears Won't Shift Smoothly (Indexing)

Drivetrain     Document TS-004     Rev. A

| Applies to | All geared Adventure Works models |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 20 – 40 minutes |
| Tools needed | Hex keys, Cable cutters (if replacing cable), Repair stand (recommended) |

## Symptoms

- Slow, hesitant, or noisy shifts that don't land cleanly on a cog.

- The chain rattles or 'chatters' as if caught between two gears.

- It shifts fine in one direction but not the other.

- It will not reach the largest or smallest cog.

Figure 1. Indexing aligns the derailleur precisely under each cog.

## Likely Causes

- Incorrect cable tension after stretch or a previous adjustment.

- High/low limit screws set wrong, blocking access to end cogs.

- Dirty, kinked, or corroded shift housing adding friction.

- Bent derailleur hanger.

- B-tension set so the guide pulley is too far from the cogs.

Safety first - Keep fingers away from the spinning chain and chainring while pedaling by hand. - Support the bike securely so the rear wheel can spin freely. - Wear eye protection if cutting cable or housing.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Shift to the smallest rear cog and the smallest front chainring to release cable tension.

- If the chain is slow to move to a larger cog, turn the rear barrel adjuster counter-clockwise a quarter turn to add tension; turn clockwise if it overshoots.

- Pedal by hand and shift one cog at a time, adjusting a quarter turn until each shift is immediate and quiet.

- If the chain won't reach the largest or smallest cog, the limit screws likely need adjustment (technician step).

- Apply a drop of light lube where the cables enter the housing and work the shifter several times.

- Re-test through the full range; quiet, instant shifts indicate correct indexing.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Set the high (H) limit so the guide pulley aligns under the smallest cog; set the low (L) limit under the largest cog without overshooting into the spokes.

- Reset cable tension from zero: with the chain on the smallest cog, just snug the cable, then index from there.

- Inspect and, if gritty or kinked, replace the shift housing and inner wire; use sealed housing ends.

- Check and correct hanger alignment with a gauge.

- Adjust B-tension for a 5–6 mm guide-pulley-to-cog gap.

- Confirm front derailleur height (1–3 mm above the big ring) and angle parallel to the chainrings.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Shifter cable pinch bolt | 5 – 7 N·m |
| Front derailleur clamp | 5 – 7 N·m |
| FD height above big ring | 1 – 3 mm |
| Guide-pulley-to-cog gap | 5 – 6 mm |
| Barrel adjuster step | ¼ turn at a time |

Figure 2. Limit screws stop the derailleur from over-traveling into the frame or spokes.

Pro tips - Make all adjustments a quarter turn at a time and test between each. - Replace shift cables yearly if you ride in wet or dusty conditions.

## When to Visit a Service Center

- Shifting is still poor after a fresh cable and correct indexing.

- The hanger is bent and no alignment gauge is available.

- An electronic (Di2/AXS) system reports an error or won't pair.

- The shifter ratchet feels broken or won't hold tension.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.