ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Flat Tire Repair and Tube Replacement

Wheels & Tires     Document TS-005     Rev. A

| Applies to | All Adventure Works models (tubed tires) |
| --- | --- |
| Difficulty | Beginner |
| Time required | 15 – 25 minutes |
| Tools needed | Tire levers, Spare tube or patch kit, Pump with gauge, 15 mm wrench (non-QR axles) |

## Symptoms

- Tire is soft or completely flat.

- The bike feels squirmy or the rim 'clunks' over bumps.

- Air added the day before is gone by morning.

- A visible object (thorn, glass, staple) is lodged in the tread.

Figure 1. Tire, tube, rim, and valve in cross-section.

## Likely Causes

- Puncture from road or trail debris.

- Pinch flat ('snakebite') from hitting an edge at low pressure.

- A failed or aged valve.

- A worn tire casing or a protruding spoke end inside the rim.

- Rim tape that has shifted and exposed a spoke hole.

Safety first - Fully deflate the tube before removing it to avoid sudden pops. - Inflate to the pressure printed on the tire sidewall — never exceed the maximum. - Check the rim and tire for the cause before fitting a new tube, or it will fail again.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Release the brake (if rim brake) and remove the wheel using the quick-release or axle.

- Deflate fully, then use tire levers to unseat one bead and pull the tube out.

- Run your fingers carefully inside the tire and along the rim to find the cause — remove any thorn, glass, or burr.

- Confirm the rim tape covers all spoke holes and sits centered.

- Lightly inflate the new tube so it holds shape, tuck it inside the tire, and seat the valve straight.

- Work the bead back on by hand (avoid pinching the tube with levers), inflate to the sidewall pressure, and check the bead is seated evenly all around.

- Refit the wheel, re-engage the brake, and confirm it spins true and the axle is tight.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- For a patch repair, inflate the tube and locate the leak by sound or by dunking in water; mark it.

- Buff the area, apply a thin even layer of vulcanizing cement, wait until tacky, then press the patch firmly for 60 seconds.

- Inspect the casing for cuts >3 mm; boot the tire or replace it if the casing is compromised.

- Verify rim strip width and condition; replace if cracked or mispositioned.

- Set pressure for rider weight and tire width (lower for wide/tubeless, higher for narrow road).

## Torque & Specifications

| Setting | Typical value |
| --- | --- |
| Road 25–28 mm | 70 – 90 psi |
| Gravel 38–45 mm | 35 – 50 psi |
| MTB 2.2–2.5" | 22 – 30 psi |
| City/hybrid 35–42 mm | 50 – 70 psi |
| Always | Stay at or below sidewall max |

Figure 2. Seat the valve straight and check the bead is even before full inflation.

Pro tips - Carry a spare tube even if you also patch — it's faster on the road. - Mark the tire position relative to the valve so a found object points you to the casing hole.

## When to Visit a Service Center

- Repeated flats with no obvious cause (possible rim or alignment fault).

- The tire casing is cut, frayed, or showing cords.

- A tubeless tire won't seal or seat on the rim.

- The rim is dented or cracked at the bead.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.