ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Tire Won't Hold Air or Leaks Slowly

Wheels & Tires     Document TS-006     Rev. A

| Applies to | All Adventure Works models (tubed & tubeless) |
| --- | --- |
| Difficulty | Beginner – Intermediate |
| Time required | 15 – 40 minutes |
| Tools needed | Pump with gauge, Valve core tool, Soapy water, Tubeless sealant (tubeless setups) |

## Symptoms

- Pressure drops noticeably over a few hours or overnight.

- Bubbles appear at the valve or along the bead when soapy water is applied.

- A tubeless tire 'burps' air when cornering hard.

- The valve hisses when you press the pump on or off.

Figure 1. Air can escape at the valve, the bead, or through the casing.

## Likely Causes

- A tiny puncture too small to find by sight.

- A loose or failing valve core.

- Dried-out tubeless sealant no longer plugging pores.

- A poor bead seal letting air escape at the rim.

- A damaged valve seal or, on tubes, a perished valve base.

Safety first - Do not exceed the tire's maximum pressure while hunting for leaks. - Keep fingers clear when seating a tubeless bead — it can seat with a sharp pop. - Tubeless sealant can stain; wear gloves and protect surfaces.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Inflate the tire and brush soapy water over the whole tire, bead, and valve; watch for growing bubbles.

- If bubbles form at the valve, check the valve core is tight using a valve-core tool.

- For tubed tires, if the leak is in the casing, replace or patch the tube (see TS-005).

- For tubeless, add fresh sealant through the valve, reinflate, and rotate/shake the wheel to spread it across the leak.

- Re-seat a leaking bead by deflating, adding a little soapy water to the bead, and inflating briskly until both beads pop into place.

- Leave the tire inflated and re-check pressure after a few hours to confirm the fix.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Replace a worn or cracked valve core; on tubeless valves, check the rubber grommet and lockring seal.

- Refresh tubeless sealant on a 2–4 month cycle (sooner in hot, dry climates).

- Inspect rim tape for lifting or punctures at spoke holes; re-tape with the correct width if compromised.

- Confirm the tire and rim are tubeless-compatible and the bead diameter matches.

- For a stubborn casing weep, install a tubeless plug or fit a tube as a roadside fix.

## Torque & Specifications

| Setting | Specification |
| --- | --- |
| Tubeless sealant refresh | Every 2 – 4 months |
| Valve core | Snug by hand + ¼ turn tool |
| Tubeless seating pressure | Up to sidewall max |
| Soap test pressure | At or below max only |
| Tape width | Match internal rim width |

Figure 2. Fresh sealant plugs small casing pores that slow-leak air.

Pro tips - Top up tubeless sealant before it fully dries to avoid clumps. - A few drops of soapy water at the bead make seating much easier.

## When to Visit a Service Center

- A tubeless tire repeatedly fails to seal despite fresh sealant.

- The rim bead seat is damaged or out of round.

- The valve cannot be sealed at the rim.

- You lack a high-volume pump or compressor to seat the bead.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.