ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Wheel Wobble, Out-of-True, or Loose Spokes

Wheels & Tires     Document TS-009     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Intermediate – Advanced |
| Time required | 30 – 60 minutes |
| Tools needed | Spoke wrench (correct nipple size), Truing stand or zip-tie pointer, Spoke tension meter (ideal) |

## Symptoms

- The rim moves side-to-side or up-and-down as the wheel spins.

- A rim brake rubs in one spot each revolution.

- A spoke 'pings' or feels loose when squeezed against its neighbor.

- A wobble that has grown worse after an impact.

Figure 1. Even spoke tension keeps the rim laterally and radially true.

## Likely Causes

- One or more spokes have lost tension.

- A spoke has loosened or broken after an impact.

- Uneven tension from the factory or a prior repair.

- A bent rim from a hard hit (pothole, curb, rock).

- A loose hub bearing letting the wheel rock.

Safety first - A broken or very loose spoke can collapse a wheel — don't ride hard until fixed. - Make small adjustments; over-tightening can crack the rim or pull the nipple through. - Check that the wheel is dished correctly (centered) before final tensioning.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Spin the wheel and watch the rim against the brake pad or a fixed pointer to find the worst wobble.

- Squeeze pairs of spokes around the wheel to feel for any that are noticeably loose.

- To pull the rim toward one side at a wobble, tighten the spokes on the opposite side a quarter turn (and/or loosen the near side).

- Work in small quarter-turn steps and re-check after each, blending corrections over several spokes.

- If a spoke is broken, fit a replacement of the correct length and bring it up to match neighboring tension.

- Re-check brake clearance and that the wheel runs true before riding.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Mount the wheel in a truing stand; correct lateral true first, then radial (hop), then re-check lateral.

- Balance spoke tension with a tension meter; aim for even readings within the rim maker's range.

- Verify dish so the rim is centered between the locknuts.

- Stress-relieve the wheel (squeeze paired spokes) and re-true to settle the build.

- Inspect for a cracked rim, damaged nipple eyelets, or a bent hub flange and replace as needed.

## Torque & Specifications

| Setting | Specification |
| --- | --- |
| Adjustment step | ¼ turn at a time |
| Lateral true target | ≤0.5 mm |
| Radial (hop) target | ≤0.5 mm |
| Spoke tension | Per rim spec (often 100–120 kgf rear DS) |
| Dish | Rim centered between locknuts |

Figure 2. Tighten opposite-side spokes to pull the rim back to center.

Pro tips - If a spoke breaks far from home, partially unthread the spoke's nipple or tape the loose spoke and ride gently to safety. - A drop of spoke-prep on threads keeps nipples from loosening over time.

## When to Visit a Service Center

- The rim is cracked, dented, or won't true within range.

- Multiple spokes are broken or the wheel is badly buckled.

- Hub bearings are loose, notchy, or have play.

- You don't have a tension meter for a high-tension wheel build.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.