ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Loose or Knocking Headset

Steering     Document TS-010     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Beginner – Intermediate |
| Time required | 15 – 30 minutes |
| Tools needed | 4 / 5 mm hex keys, Torque wrench |

## Symptoms

- A knock or clunk through the bars when braking or riding over bumps.

- Play felt at the fork when you rock the bike with the front brake held.

- Notchy or gritty steering that catches at center.

- Steering that feels stiff or won't return to center.

Figure 1. The headset bearings sit in the head tube; the top cap sets preload.

## Likely Causes

- Headset preload has loosened, leaving bearing play.

- Over-tightened preload binding the bearings.

- Worn, dry, or pitted headset bearings.

- Loose stem clamp bolts mimicking headset play.

- A loose or worn fork/steerer interface.

Safety first - The headset connects the steering to the frame — don't ride with play or binding. - Set the top cap to preload only; the stem bolts (not the top cap) clamp the steerer. - Re-check after the first short ride as the assembly settles.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Hold the front brake and rock the bike forward and back. A knock or movement at the head tube indicates play.

- Lift the front wheel and turn the bars — if steering is rough or notchy, the bearings need service, not just preload.

- Loosen the two stem clamp bolts on the steerer.

- Gently snug the top-cap bolt until the play just disappears (usually 1–2 N·m) — do not over-tighten.

- Align the stem with the front wheel and re-tighten the stem clamp bolts to spec, alternating evenly.

- Re-check: no play when rocking the bike, and the bars should turn freely and smoothly.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- If preload won't remove play, check there is a 2–4 mm gap between the top of the steerer and the top of the stem/spacers — add a spacer or trim as needed so the cap can preload.

- Disassemble, clean, and inspect the bearings; replace pitted or rusty cartridges/balls.

- Grease the bearing seats and races; check the crown race and compression ring for wear.

- Confirm the fork steerer is not cracked and is the correct length.

- Reassemble, set preload, then torque the stem to spec.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Top-cap preload bolt | Snug only, ~1 – 2 N·m |
| Stem steerer clamp bolts | 5 – 6 N·m |
| Steerer-to-stem top gap | 2 – 4 mm |
| Stem bar clamp bolts | 5 – 6 N·m |
| Bearing service | Clean & grease yearly |

Figure 2. Preload with the top cap first, then clamp the stem to the steerer.

Pro tips - Order of operations matters: preload with the cap, then tighten the stem bolts. - A faint knock only under braking is almost always headset play.

## When to Visit a Service Center

- Bearings are pitted, rusted, or notchy and need replacement.

- There is no gap above the steerer to allow preload.

- You suspect a cracked steerer or head tube.

- Play remains after correct preload and stem torque.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.