ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Saddle Height, Slipping, or Creaking Seatpost

Saddle & Fit     Document TS-012     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Beginner |
| Time required | 10 – 25 minutes |
| Tools needed | 4 / 5 mm hex key, Torque wrench, Grease or carbon paste, Tape measure |

## Symptoms

- The saddle slowly drops during a ride.

- A creak from under the saddle when pedaling.

- The saddle tilts or twists out of position.

- Knee discomfort from a saddle set too high or too low.

Figure 1. Mind the minimum-insertion mark and set the correct saddle height.

## Likely Causes

- Seatpost clamp under-torqued or the post is greasy where it shouldn't be.

- Carbon post or frame assembled without carbon paste.

- Dry saddle-rail clamp creaking under load.

- Incorrect saddle height for the rider.

- A damaged or mismatched-diameter seatpost.

Safety first - Never raise the post above the minimum-insertion line — it can crack the frame or post. - Use grease on alloy posts and carbon paste on carbon interfaces; over-greasing carbon causes slipping. - Torque the clamp to spec; over-tightening can crush a lightweight post.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Set saddle height: with the heel on the pedal at the bottom of the stroke, your leg should be straight; it bends slightly when pedaling with the ball of the foot.

- Mark or note the insertion depth, and confirm the post is past the minimum-insertion line.

- For slipping, remove the post, clean it, and apply grease (alloy) or carbon paste (carbon) to the clamp zone only.

- Reinsert to your mark, level the saddle, and torque the seatpost clamp to spec.

- For a creak, also remove, clean, and lightly grease the saddle-rail clamp bolts and re-torque.

- Sit and pedal hard to confirm the saddle holds position and is quiet.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Verify the seatpost diameter matches the frame (e.g., 27.2 / 30.9 / 31.6 mm) and isn't relying on a worn shim.

- Inspect the post and frame collar for cracks or deformation, especially around the slot.

- For dropper posts, check the actuation, air pressure, and bushing play per the maker's guide.

- Apply friction paste for carbon and confirm the clamp closes evenly without pinching.

- Set rail clamp torque to spec and confirm rails sit in the clamp's allowed zone.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Seatpost clamp | 5 – 7 N·m |
| Saddle rail clamp | 5 – 7 N·m |
| Carbon interface | Use carbon paste |
| Insertion | Past minimum mark |
| Heel-on-pedal | Leg straight at bottom |

Figure 2. Keep the post inserted past the minimum-insertion line at all times.

Pro tips - Record your saddle height and setback so you can reset your fit exactly. - Change saddle height in small 3–5 mm steps to find your comfortable position.

## When to Visit a Service Center

- The seatpost or frame collar is cracked.

- A dropper post leaks, sags, or won't return.

- The post diameter doesn't match the frame.

- Creaking persists after cleaning and greasing both clamps.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.