ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Bent Derailleur Hanger and Poor Shifting

Drivetrain     Document TS-016     Rev. A

| Applies to | All geared Adventure Works models |
| --- | --- |
| Difficulty | Intermediate – Advanced |
| Time required | 20 – 45 minutes |
| Tools needed | Derailleur hanger alignment gauge, Hex keys, Spare hanger (recommended) |

## Symptoms

- Shifting is poor across all gears even after indexing.

- The chain shifts unevenly going up vs. down the cassette.

- The rear derailleur looks angled inward toward the wheel.

- Shifting got worse right after the bike fell on its drive side.

Figure 1. The hanger holds the rear derailleur in precise alignment with the cassette.

## Likely Causes

- A bent derailleur hanger (the sacrificial bracket between frame and derailleur).

- Impact from a crash, fall, or transport knock.

- A previous over-shift driving the derailleur into the spokes.

- A loose or damaged hanger bolt.

- A bent derailleur cage (separate from the hanger).

Safety first - Never shift into the largest cog if the hanger may be bent toward the spokes — the derailleur can enter the wheel. - The hanger is designed to bend or break to protect the frame; don't force a badly bent one straight repeatedly. - Check alignment before any hard riding after an impact.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Sight from behind the bike: the rear derailleur and its jockey wheels should sit vertically, parallel to the cassette cogs.

- If the derailleur leans toward the wheel, the hanger is likely bent.

- Carry and fit a spare hanger if you have the correct model — replacement is more reliable than bending.

- After fitting a straight hanger, re-index the rear shifting (see TS-004).

- Test shifts in a safe area, paying attention to the largest cog for clearance.

- If shifting is still off, alignment should be checked with a gauge (technician step).

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Thread a hanger alignment gauge into the derailleur mount and measure the rim gap at multiple clock positions.

- Gently correct the hanger so the gap is equal all around (alloy hangers can be bent back; replace if cracked).

- Confirm the correct hanger model — hangers are frame-specific.

- Re-set limit screws, B-tension, and cable indexing after alignment.

- Inspect the derailleur cage and pulleys for damage and replace if bent or worn.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Hanger bolt(s) | 4 – 6 N·m |
| Derailleur mount bolt | 8 – 10 N·m |
| Alignment target | Equal rim gap all positions |
| B-screw gap | 5 – 6 mm |
| Hanger type | Frame-specific — match exactly |

Figure 2. A bent hanger tilts the derailleur and ruins indexing in every gear.

Pro tips - Keep a spare hanger for your exact frame — it's the cheapest insurance against a ruined ride. - After any drive-side fall, check hanger alignment before forcing a shift.

## When to Visit a Service Center

- You don't have an alignment gauge or the correct spare hanger.

- The hanger is cracked or the threads are damaged.

- The derailleur body or parallelogram is bent.

- Shifting remains poor after alignment and re-indexing.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.