ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# E-Bike Won't Power On or Loses Range

E-Bike     Document TS-019     Rev. A

| Applies to | PowerRide e-Bike, City Cruiser-E |
| --- | --- |
| Difficulty | Beginner – Intermediate |
| Time required | 15 – 40 minutes |
| Tools needed | Manufacturer charger, Clean dry cloth, Soft brush for contacts |

## Symptoms

- The display or system won't turn on.

- The motor cuts out intermittently while riding.

- Range per charge has dropped noticeably.

- A battery error code appears on the display.

Figure 1. The battery, controller, and motor form the e-bike's power system.

## Likely Causes

- A flat or deeply discharged battery.

- The battery isn't fully seated or locked into its mount.

- Dirty or corroded battery/controller contacts.

- Cold temperatures temporarily reducing capacity.

- Battery aging or a charger fault.

Safety first - Use only the manufacturer's charger; the wrong charger is a fire risk. - Never charge a visibly damaged, swollen, or wet battery — isolate it and contact support. - Charge on a hard, non-flammable surface and don't leave charging unattended overnight.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Confirm the battery is charged: connect the official charger and watch for the charging indicator.

- Remove and reseat the battery, making sure it clicks fully into its lock.

- Inspect the battery and frame contacts; gently clean any dirt or moisture with a dry cloth/soft brush.

- Power-cycle the system: turn it fully off, wait 10 seconds, and turn it back on.

- Note any error code shown and check it against the owner's manual.

- If cold, let the battery warm to room temperature before judging range or charge.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Check the charger output and the charge port for damage or bent pins.

- Inspect the wiring harness connectors from battery to controller to motor for corrosion or looseness.

- Read diagnostic/error codes via the display or the maker's tool and follow the indicated procedure.

- Assess battery health/state-of-charge cycles if the tool reports it; aging cells reduce range.

- Verify firmware is current and assist levels/settings are correct for expected range.

## Torque & Specifications

| Item | Guidance |
| --- | --- |
| Charger | Manufacturer's unit only |
| Storage charge | ~40 – 60% for long storage |
| Charge temperature | 10 – 30 °C ideal |
| Contacts | Keep clean & dry |
| Error codes | Cross-check owner's manual |

Figure 2. Follow the maker's battery and charging guidance precisely for safety.

Pro tips - Store the battery around 40–60% charge in a cool, dry place to slow aging. - Cold weather temporarily cuts range — capacity returns as the battery warms.

## When to Visit a Service Center

- The battery is swollen, damaged, leaking, or very hot.

- An error code points to the motor or controller.

- Range has fallen far below spec on a healthy charge.

- The charger or charge port is damaged.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.