ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Chain Skipping or Slipping Under Load

Drivetrain     Document TS-003     Rev. A

| Applies to | All geared Adventure Works models |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 20 – 45 minutes |
| Tools needed | Chain wear gauge, Chain tool, Hex keys, Cassette lockring tool & chain whip |

## Symptoms

- The chain jumps or 'ghost-shifts' when you pedal hard, especially uphill.

- A rhythmic clunk once per pedal stroke in certain gears.

- Skipping is worse in the most-used gears (middle of the cassette).

- Shifting that was once crisp has become vague.

Figure 1. Chain, cassette, and rear derailleur must work together for clean power transfer.

## Likely Causes

- A worn chain that has 'stretched' past 0.5–0.75% elongation.

- Worn cassette cogs that no longer mesh with a new chain.

- Cable tension drift causing the chain to ride between cogs.

- A stiff or seized chain link from rust or a previous repair.

- Bent derailleur hanger throwing off indexing.

Safety first - Keep fingers clear of the chain and chainring — a chain under load can pinch hard. - Work with the bike securely supported in a stand or stable on its wheels. - Degreasers and lubricants can make floors slippery; clean up spills immediately.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Shift through every gear while pedaling lightly and note which gears skip.

- Check the chain with a wear gauge. If a 0.5% gauge drops fully into the chain, the chain is worn and should be replaced.

- Inspect for a stiff link: back-pedal slowly and watch for a link that hops as it passes the derailleur; flex it side-to-side to free it.

- Fine-tune cable tension with the rear barrel adjuster — a quarter turn at a time — until the chain runs quietly in each gear.

- Clean and re-lubricate the chain, then wipe off excess lube.

- Re-test under load by pedaling firmly while seated. If skipping continues, the cassette is likely worn (technician step).

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Measure chain elongation precisely; replace at 0.5% (11/12-speed) or 0.75% (≤10-speed).

- If the chain was ridden well past the wear limit, replace the cassette together with the chain — a new chain on a worn cassette will keep skipping.

- Check derailleur hanger alignment with a gauge; straighten or replace if bent.

- Verify B-tension so the upper jockey wheel clears the largest cog by 5–6 mm.

- Set the high/low limit screws, then index the shifting from the smallest cog outward.

- Confirm the chain is the correct length: in the big-big combination the derailleur cage should hang just short of horizontal.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Chain replacement limit | 0.5% (11/12-spd), 0.75% (≤10-spd) |
| Cassette lockring | 40 N·m |
| Derailleur hanger bolt | 5 – 6 N·m |
| Jockey-to-cog gap (B-screw) | 5 – 6 mm |
| Rear derailleur mount bolt | 8 – 10 N·m |

Figure 2. A worn chain rounds the cogs; replace both if elongation is severe.

Pro tips - Replace the chain before it reaches 0.5% wear to make the cassette last far longer. - Number your chain quick-link orientation so reinstalls are consistent.

## When to Visit a Service Center

- Skipping continues after fitting a new chain and cassette.

- The derailleur hanger is bent and you lack an alignment gauge.

- Shifting is erratic across all gears (possible worn shifter or frayed cable).

- There is play or roughness in the freehub body.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.