ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Diagnosing Creaks, Clicks, and Knocks

Frame & Noise     Document TS-007     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 30 – 60 minutes |
| Tools needed | Hex & Torx keys, Torque wrench, Bike-specific grease & anti-seize, Clean rags |

## Symptoms

- A creak that appears with each pedal stroke.

- A click felt through the bars, saddle, or pedals.

- Noise that changes when you stand vs. sit, or in/out of the saddle.

- A knock over bumps that you can't localize.

Figure 1. Work methodically through each interface to isolate the noise.

## Likely Causes

- Dry or under-torqued interface (pedals, crank, seatpost, stem, bottle cage).

- A loose bottom bracket or headset.

- Chainring or cassette bolts that have loosened.

- Dry seatpost or saddle rail clamp.

- Surprising culprits: water bottle cage, wheel skewer, or even shoe cleats.

Safety first - Re-torque every fastener to spec — over-tightening can crack carbon or strip alloy threads. - Use the correct grease or carbon assembly paste for each interface. - Recheck critical bolts (stem, seatpost, pedals) before riding.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Try to reproduce the noise: note exactly when it happens (pedaling, bumps, braking, standing).

- Eliminate the obvious: remove the water bottle and cage and ride — many 'frame creaks' are a loose cage.

- Check pedal tightness; remove, grease the threads, and reinstall to spec (remember the left pedal is reverse-threaded).

- Confirm the seatpost is greased (or has carbon paste) and the clamp is at spec; a dry post is a classic creak source.

- Tighten the stem and chainring bolts to spec, one at a time.

- If the noise persists with pedaling under load, suspect the bottom bracket or crank (technician step).

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Isolate by section: pedal off the bike to test the BB/crank; coast to test the wheels/drivetrain.

- Remove, clean, grease, and re-torque the crank and bottom bracket; replace a notchy or play-laden BB.

- Service the headset bearings and set preload so there is no play but the bars rotate freely.

- Check chainring bolts, cassette lockring, and pedal spindles for play.

- Inspect frame interfaces (cable guides, derailleur hanger, seatpost collar) and apply appropriate paste/grease.

- Confirm wheel skewers/thru-axles and dropout faces are clean and torqued.

## Torque & Specifications

| Fastener | Torque |
| --- | --- |
| Pedals | 35 N·m |
| Seatpost clamp | 5 – 7 N·m |
| Stem (bar & steerer) | 5 – 6 N·m |
| Chainring bolts | 8 – 10 N·m |
| Bottle cage bolts | 3 – 4 N·m |

Figure 2. Correct torque eliminates most creaks and protects components.

Pro tips - Change one thing at a time and test, or you'll never know which fix worked. - Most creaks are an unlubricated, correctly-or-incorrectly torqued interface — not a broken part.

## When to Visit a Service Center

- The noise persists after servicing pedals, seatpost, BB, and headset.

- You suspect a cracked frame or a failing bearing.

- A press-fit bottom bracket needs special tools to remove.

- Carbon components are involved and you're unsure of safe torque.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.