ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Chain Keeps Dropping Off the Chainring

Drivetrain     Document TS-008     Rev. A

| Applies to | All geared Adventure Works models |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 20 – 40 minutes |
| Tools needed | Hex keys, Torque wrench, Chain checker |

## Symptoms

- The chain falls to the inside (toward the frame) when shifting to the small ring.

- The chain drops to the outside when shifting to the big ring.

- Chain drops over bumps even without shifting.

- A grinding sound as the chain rubs the frame after dropping.

Figure 1. Front shifting and ring profile control whether the chain stays put.

## Likely Causes

- Front derailleur limit screws set too wide.

- Front derailleur height or angle out of adjustment.

- A worn or stretched chain riding poorly on the rings.

- Worn chainring teeth (shark-fin profile).

- Missing or misadjusted chain catcher on 1x setups.

Safety first - Stop pedaling immediately if the chain drops to avoid jamming it in the frame. - Keep fingers clear of the chainring teeth. - Support the bike so you can turn the cranks safely by hand.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Reproduce the drop and note the direction — inside drops mean the low limit is too loose; outside drops mean the high limit is too loose.

- For inside drops, turn the front derailleur 'L' limit screw clockwise a little to stop the cage swinging too far in.

- For outside drops, turn the 'H' limit screw clockwise a little to limit outward travel.

- Shift across the range and fine-tune so the chain reaches both rings without dropping.

- Check chain wear; replace a chain past its limit, as a worn chain drops more easily.

- On a 1x drivetrain, confirm the chain-catcher or clutch is functioning.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Set front derailleur height to 1–3 mm above the big-ring teeth and align the cage parallel to the rings.

- Re-set both limit screws precisely, then re-index the front shift.

- Inspect chainrings for worn or hooked teeth; replace worn rings.

- On 1x, verify the narrow-wide ring is in good condition and the rear derailleur clutch holds adequate tension.

- Fit or adjust a chain catcher just inboard of the small ring as insurance.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| FD height above big ring | 1 – 3 mm |
| FD clamp bolt | 5 – 7 N·m |
| Chainring bolts | 8 – 10 N·m |
| Chain catcher bolt | 2 – 3 N·m |
| Chain wear limit | 0.5 – 0.75% |

Figure 2. Limit screws define how far the chain can travel in each direction.

Pro tips - A quick win on 1x bikes is fitting a simple chain catcher. - Avoid extreme cross-chaining (big-big / small-small) to reduce drops and wear.

## When to Visit a Service Center

- Chainrings are visibly worn and need replacement.

- A 1x clutch derailleur no longer holds tension.

- The front derailleur mount or braze-on is damaged.

- Drops continue after correct limit and height setup.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.