ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Handlebar or Stem Slipping or Misaligned

Steering     Document TS-011     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Beginner |
| Time required | 10 – 20 minutes |
| Tools needed | 4 / 5 mm hex keys, Torque wrench, Carbon assembly paste (carbon parts) |

## Symptoms

- The handlebar rotates down (or up) under pressure.

- The bars are not square to the front wheel.

- The brake levers or shifters twist on the bar.

- A creak from the bar/stem area when pulling on the bars.

Figure 1. The stem clamps the bar at the face-plate and the steerer at the rear.

## Likely Causes

- Stem face-plate or clamp bolts under-torqued.

- Greasy or contaminated clamp surfaces letting parts slip.

- Carbon bar/stem assembled without carbon paste.

- Uneven face-plate gap loading the bar on one side.

- Worn or damaged clamp surfaces.

Safety first - Loose bars can rotate suddenly and cause a crash — torque correctly before riding. - Use carbon paste, not grease, on carbon interfaces to prevent over-tightening. - Never exceed the printed torque on the stem or bar; over-torque can crack them.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Loosen the face-plate bolts and align the bar to the desired angle (brake levers usually angled so wrists are neutral).

- Square the stem to the front wheel: grip the wheel between your knees and align the stem, then snug the steerer clamp bolts.

- Tighten the four face-plate bolts in a cross pattern, a little at a time, keeping the gap even top and bottom.

- Torque all bolts to the values printed on the stem (typically 5–6 N·m).

- Confirm an even gap on the face-plate and that the levers don't twist under firm hand pressure.

- Test by pressing down hard on the bars — they should not move.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Degrease clamp surfaces; apply carbon assembly paste to carbon bar/stem interfaces to raise friction without extra torque.

- Inspect the bar clamp area for cracks, gouges, or crush marks; replace if damaged.

- Confirm the bar diameter matches the stem (e.g., 31.8 vs 35 mm) and that shims aren't masking a mismatch.

- Set lever/shifter clamp torque to spec so they slip in a crash rather than break.

- Re-check after a short ride and re-torque if needed.

## Torque & Specifications

| Fastener | Torque |
| --- | --- |
| Stem face-plate bolts | 5 – 6 N·m (even gap) |
| Stem steerer bolts | 5 – 6 N·m |
| Brake/shifter clamp | 5 – 8 N·m |
| Carbon interfaces | Use carbon paste |
| Tightening pattern | Cross / alternating |

Figure 2. Even, correct torque keeps the cockpit secure and protects components.

Pro tips - An even face-plate gap top and bottom spreads the load and prevents bar damage. - Mark the bar angle with tape before loosening so you can return to your fit.

## When to Visit a Service Center

- The bar or stem clamp is cracked or gouged.

- A carbon bar has slipped repeatedly despite correct paste and torque.

- Bar and stem diameters don't match.

- Levers continue to twist after correct clamp torque.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.