ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Pedals Creaking, Loose, or Cleat Problems

Drivetrain     Document TS-013     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Beginner |
| Time required | 10 – 25 minutes |
| Tools needed | 8 mm hex key / 15 mm pedal wrench, Hex keys for cleats, Grease, Torque wrench |

## Symptoms

- A creak or click once per pedal stroke.

- Play felt at the pedal when rocked by hand.

- Clipless pedals are hard to clip into or release.

- A clunk when putting power down.

Figure 1. Pedals thread into the crank arms at the bottom bracket.

## Likely Causes

- Dry pedal threads creaking against the crank.

- Loose pedal that wasn't torqued in.

- Worn cleats causing inconsistent engagement.

- Cleat bolts loosened on the shoe.

- Worn pedal bearings with play.

Safety first - Remember the left pedal is reverse (left-hand) threaded — turn it the 'wrong' way to remove. - A loose pedal can damage the crank threads; torque to spec. - Practice clipping out at a wall before riding clipless in traffic.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Rock each pedal by hand to feel for bearing play, and listen for a creak as you pedal under load.

- Remove both pedals (left is reverse-threaded), clean and grease the threads, and reinstall to spec.

- Check cleat bolts on the shoe and tighten any that have loosened; align cleats to your natural foot angle.

- If engagement is poor, inspect cleats for wear and replace worn cleats.

- Adjust clipless release tension (spring screw) lighter for easier exit while learning.

- Test that pedals spin freely without play and engage/release cleanly.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Service or replace pedal bearings if there is play or roughness after greasing the threads.

- Confirm cleat shims/wedges if the rider has a leg-length or alignment need.

- Check crank thread condition; chase damaged threads or replace the crank arm if stripped.

- Match cleat type to pedal (e.g., SPD vs SPD-SL) and set float to rider preference.

- Verify Q-factor and pedal washer use on cranks that require them.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Pedal into crank | 35 N·m |
| Cleat bolts | 5 – 6 N·m |
| Left pedal thread | Reverse (LH) |
| Release tension | Lighter for beginners |
| Bearing play | None permitted |

Figure 2. Greased, correctly torqued pedal threads eliminate most pedal creaks.

Pro tips - A tiny amount of grease on cleat bolts stops the most common 'pedal' creak — which is often the cleat. - Replace cleats before they're badly worn to keep release predictable.

## When to Visit a Service Center

- Pedal bearings are worn and the pedal isn't user-serviceable.

- Crank threads are stripped or cross-threaded.

- Persistent knee pain suggests a professional bike fit.

- Clip-in/out remains inconsistent after new cleats.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.