ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Bottom Bracket or Crank Play and Creaking

Drivetrain     Document TS-014     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Advanced |
| Time required | 30 – 75 minutes |
| Tools needed | BB tool (model-specific), Crank tools / hex keys, Torque wrench, Grease / anti-seize / retaining compound |

## Symptoms

- Side-to-side play felt by rocking the crank arms.

- A deep creak under hard pedaling, often when standing.

- Grinding or roughness when turning the cranks by hand.

- A knock that tracks with pedal position, not wheel position.

Figure 1. The bottom bracket carries the crank axle at the center of the frame.

## Likely Causes

- Loose crank preload or pinch bolts.

- Worn or dry bottom bracket bearings.

- A press-fit BB that has loosened in the shell.

- Contaminated or dry crank/BB interface.

- Incorrect spacers causing bearing bind or play.

Safety first - Use the correct BB tool for your standard to avoid rounding the cups. - Follow the maker's torque and assembly compound guidance, especially for press-fit. - Recheck crank preload and pinch-bolt torque after the first ride.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Rock the crank arms side to side to feel for play, and spin them to feel for roughness.

- On a two-piece crank, snug the preload cap until play disappears, then torque the arm pinch bolts to spec.

- Confirm the issue isn't actually the pedals or chainring bolts (check those first — they're easier).

- If there's roughness or persistent creak, the bearings likely need service (technician step).

- Keep the BB area clean; avoid blasting it with a pressure washer.

- Re-test for play and noise after tightening.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Remove the crank and BB; identify the standard (threaded BSA/Italian vs press-fit BB30/PF30/BB86, etc.).

- Inspect bearings for play, notchiness, or rust; replace worn bearings/cartridges.

- Clean shell and threads/bores; apply grease or anti-seize on threaded BBs, or the specified retaining compound on press-fit.

- Install with the correct spacers and preload so bearings spin free without play.

- Torque threaded cups to spec; for press-fit, press squarely and check for a creak-free fit (consider a thread-together BB if creaks persist).

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Threaded BB cups | 35 – 50 N·m |
| Crank preload cap | Snug, ~0.5 – 2 N·m |
| Crank arm pinch bolts | 12 – 14 N·m |
| Bearing play | None permitted |
| Press-fit | Use specified compound |

Figure 2. Preload first, then torque the pinch bolts; respect press-fit procedures.

Pro tips - Persistent press-fit creak? A thread-together BB often solves it permanently. - Always set crank preload before tightening the pinch bolts.

## When to Visit a Service Center

- A press-fit BB needs press/extraction tools you don't have.

- The BB shell or frame threads are damaged.

- Creaking continues after a full BB service.

- You're unsure which BB standard your frame uses.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.