ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Squeaky, Dry Chain — Cleaning and Lubrication

Maintenance     Document TS-015     Rev. A

| Applies to | All Adventure Works models |
| --- | --- |
| Difficulty | Beginner |
| Time required | 15 – 30 minutes (plus dry time) |
| Tools needed | Chain lube (wet or dry), Degreaser, Brush or chain cleaner, Clean rags |

## Symptoms

- A squeak or chirp from the chain that rises with pedaling speed.

- A dry, rusty, or grimy-looking chain.

- Sluggish shifting and extra drivetrain noise.

- Black, gritty buildup on the cassette and chainrings.

Figure 1. A clean, properly lubricated chain runs quietly and lasts longer.

## Likely Causes

- Lubricant has worn off, leaving metal-on-metal contact.

- Riding in rain or dust stripped or contaminated the lube.

- Over-lubing attracted grit that grinds the chain.

- Using the wrong lube for conditions (dry lube in wet, or vice versa).

- A long-neglected chain starting to surface-rust.

Safety first - Degreasers can be slippery and irritating — wear gloves and protect the floor. - Keep lube and degreaser away from brake rotors, pads, and rims. - Let the chain dry before applying lube, and wipe excess after.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Backpedal the chain through a rag to wipe off loose grime, then through a brush or chain cleaner with degreaser.

- Rinse lightly if using a wash-off degreaser, then dry the chain completely with a clean rag.

- Apply one drop of lube to each roller while backpedaling slowly — a full revolution or two.

- Let the lube soak in for the time on the bottle (often 5–15 minutes, or longer for wax).

- Wipe off all excess lube from the outside of the chain; only the inside needs it.

- Shift through the gears to distribute lube, then wipe once more.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Match lube to conditions: dry/wax lube for dry/dusty riding, wet lube for rain and long wet rides.

- For a deep clean, remove the chain via the quick-link and soak/agitate in degreaser, then dry fully.

- Inspect chain wear while clean; replace at the wear limit.

- Clean the cassette, chainrings, and jockey wheels — fresh lube on a dirty drivetrain wears fast.

- Consider a wax-based system for the cleanest, longest-lasting drivetrain if the rider maintains the routine.

## Torque & Specifications

| Item | Guidance |
| --- | --- |
| Re-lube interval | Every 100–200 km or after rain |
| Lube amount | One drop per roller |
| Dry time | Per bottle (5 min – overnight wax) |
| Dry lube | Dry/dusty conditions |
| Wet lube | Wet/muddy conditions |

Figure 2. Apply lube to the inner rollers and wipe the outside clean.

Pro tips - Less is more — a wiped-down chain stays cleaner and quieter than a dripping one. - Re-lube after every wet ride to prevent rust forming overnight.

## When to Visit a Service Center

- The chain is rusted into stiff links that won't free up.

- The drivetrain is worn and needs replacement.

- You want a professional ultrasonic clean and wax treatment.

- Noise persists after cleaning and correct lubrication.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.