ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Warped Rotor or Contaminated Disc Pads

Brakes     Document TS-017     Rev. A

| Applies to | Trailblazer MTB, Summit Trail, Velocity Road (disc), PowerRide e-Bike |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 20 – 45 minutes |
| Tools needed | Rotor truing fork, Isopropyl alcohol & clean rag, Sandpaper (deglaze), Torque wrench |

## Symptoms

- A pulsing or 'wow-wow' feel through the lever when braking.

- Greatly reduced braking power, sometimes with a glassy lever feel.

- A persistent rub that returns no matter how you center the caliper.

- A burnt or oily smell from the brake after washing or lubing.

Figure 1. A true, clean rotor and uncontaminated pads are essential for power.

## Likely Causes

- A bent/warped rotor from heat or an impact.

- Pads contaminated by oil, degreaser, or chain lube.

- Glazed pads from prolonged dragging.

- Loose rotor bolts allowing movement.

- Overheating on a long descent.

Safety first - Contaminated pads can't always be cleaned — replace them if power doesn't return. - Rotors get extremely hot; let them cool before handling. - Keep all lubricants and sprays away from the braking system.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Spin the wheel and watch the rotor pass through the caliper — a side-to-side wobble indicates a warp.

- Clean the rotor with isopropyl alcohol and a clean rag; never use an oily cloth.

- If braking is weak with a glassy feel, the pads may be contaminated or glazed.

- Lightly deglaze pads by rubbing them on fine sandpaper laid flat; replace if oil has soaked in.

- Re-bed the brakes with 20–30 firm stops from moderate speed.

- If a pulsing remains, the rotor needs truing (technician step).

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Check rotor runout with a truing fork; gently bend high spots back to within 0.2 mm.

- Re-torque 6-bolt rotors in a star pattern or check the Center-Lock lockring.

- Replace oil-contaminated pads — burning them off is unreliable and risky.

- Confirm the caliper pistons advance and retract evenly; clean and exercise sticky pistons.

- For repeated overheating, advise larger rotors or improved braking technique on descents.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Max rotor runout | 0.2 mm |
| 6-bolt rotor bolts | 4 – 6 N·m (star) |
| Center-Lock lockring | 40 N·m |
| Min. rotor thickness | Per rotor marking (often 1.5 mm) |
| Caliper mount bolts | 6 – 8 N·m |

Figure 2. Even rotor runout and clean pads restore consistent stopping power.

Pro tips - Buy a spare set of pads before long trips — contamination can end a ride. - Wash the bike with rotors facing away from spray and degreaser.

## When to Visit a Service Center

- The rotor is below minimum thickness or cracked.

- Pads are contaminated and replacement still gives weak braking.

- A caliper piston is seized.

- You lack a truing fork for a warped rotor.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.