ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Suspension Fork Not Working or Setting Sag

Suspension     Document TS-018     Rev. A

| Applies to | Trailblazer MTB, Summit Trail (suspension models) |
| --- | --- |
| Difficulty | Intermediate |
| Time required | 20 – 40 minutes |
| Tools needed | Shock pump, Sag measurement (zip tie / o-ring), Hex keys |

## Symptoms

- The fork feels harsh and barely moves over bumps.

- The fork dives too easily or bottoms out hard.

- Uneven or sticky travel with a 'clunk'.

- Oil weeping from the seals.

Figure 1. The suspension fork connects the front wheel to the cockpit.

## Likely Causes

- Incorrect air pressure for the rider's weight.

- The lockout or compression damping is left fully closed.

- Low air volume or lost pressure over time.

- Dry stanchions/seals needing service.

- Worn seals leaking oil and air.

Safety first - Use a proper shock pump with a gauge — never a tire pump or compressor on an air fork. - Release pressure slowly; air forks hold high pressure. - Don't ride with leaking seals — internal damage can follow.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Confirm the lockout/compression lever is fully open before testing the fork's movement.

- Set sag: fit a zip tie or use the o-ring on the stanchion, sit on the bike in riding position, and measure how far it compresses.

- Aim for roughly 15–20% sag for cross-country and 20–30% for trail riding (check your fork's chart).

- Adjust air pressure with a shock pump in small steps to reach the target sag — more pressure for less sag.

- Set rebound so the fork returns quickly but doesn't 'pogo'.

- Wipe the stanchions clean and add a drop of suspension oil to keep seals supple.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Perform a lower-leg service: clean, inspect, and re-oil the seals and foam rings at the maker's interval.

- Check air-spring pressure holds over 24 hours; a drop indicates a failing seal or valve.

- Inspect stanchions for scratches that damage seals; replace seals if oil is weeping.

- Verify token/volume-spacer count to tune bottom-out resistance.

- Set compression and rebound from the maker's baseline for rider weight, then fine-tune on terrain.

## Torque & Specifications

| Setting | Guidance |
| --- | --- |
| XC sag | 15 – 20% of travel |
| Trail sag | 20 – 30% of travel |
| Air tool | Shock pump only |
| Lower-leg service | Every 50 hrs (typical) |
| Pressure check | Hold over 24 hrs |

Figure 2. Set sag with the rider in normal riding position, then tune rebound.

Pro tips - Note your air pressure and clicks once dialed in so you can reset after travel. - A quick stanchion wipe before each ride dramatically extends seal life.

## When to Visit a Service Center

- Seals are leaking oil or the fork loses pressure quickly.

- Internal damping needs a full service or rebuild.

- Stanchions are scratched or pitted.

- The fork still feels harsh or uneven after sag and rebound setup.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.