ADVENTURE WORKS  ·  TECHNICAL SUPPORT

# Brake Lever and Cable Adjustment; Replacing Worn Pads

Brakes     Document TS-020     Rev. A

| Applies to | All Adventure Works models with cable-pull or rim brakes |
| --- | --- |
| Difficulty | Beginner – Intermediate |
| Time required | 20 – 40 minutes |
| Tools needed | Hex keys, Cable cutters, Torque wrench, New pads (as needed) |

## Symptoms

- Lever reach is too far or too close for comfortable braking.

- Brakes engage too late or feel under-powered.

- Pads are worn to or past the wear indicator.

- Uneven pad wear or one pad touching first.

Figure 1. Pad wear and cable tension both affect lever feel and power.

## Likely Causes

- Lever reach not adjusted to hand size.

- Cable slack from stretch or a loose pinch bolt.

- Worn pads reducing bite and increasing lever travel.

- Caliper not centered, causing uneven pad contact.

- Frayed or sticky cable/housing adding friction.

Safety first - Test braking at walking pace before riding — both brakes must be firm and reliable. - Replace pads before the metal backing or wear line reaches the rim/rotor. - Keep oil and lube off pads and braking surfaces.

## Customer Fix — Step by Step

Suitable for home repair with basic tools. Work through the steps in order.

- Set lever reach using the reach-adjust screw so you can brake firmly without overstretching your fingers.

- Inspect pads: rim pads have wear grooves; replace at ~1 mm. Disc pads need ≥1 mm of material above the backing.

- To replace pads, note the orientation, remove the retaining bolt/pin, fit the new pads, and align them to the braking surface.

- For rim brakes, set a 1–2 mm pad gap and toe-in the leading edge slightly to reduce squeal.

- Take up cable slack with the barrel adjuster, or loosen the pinch bolt, pull the cable taut, and re-torque it.

- Center the caliper, then test that both pads contact evenly and the lever is firm.

## Technician Reference

Additional checks and adjustments for service technicians and advanced users.

- Replace gritty or frayed cables and housing; use sealed ferrules and a smooth housing run.

- Set pad toe-in (rim) to ~0.5 mm leading-edge contact to quiet the brake.

- Center the caliper precisely and equalize spring tension on dual-pivot rim calipers.

- Bed in new pads with 20–30 progressive stops.

- Confirm even pad-to-surface contact across the full pad face and correct final lever throw.

## Torque & Specifications

| Fastener / setting | Specification |
| --- | --- |
| Cable pinch bolt | 5 – 7 N·m |
| Pad-fixing bolt | 5 – 7 N·m |
| Rim pad gap | 1 – 2 mm each side |
| Rim pad toe-in | ~0.5 mm leading edge |
| Minimum pad material | 1 mm |

Figure 2. Torque pad and cable bolts to spec and bed in new pads before riding.

Pro tips - Match pad compound to your braking surface (alloy vs. carbon rim; resin vs. metallic for discs). - Replace cables annually if you ride in wet or dusty conditions for the crispest feel.

## When to Visit a Service Center

- Hydraulic brakes that need a bleed rather than cable adjustment.

- The caliper won't center or a spring is broken.

- Pads wear unevenly again soon after replacement.

- You're unsure which pad compound suits your rim or rotor.

This guide is provided for reference. Torque values are typical starting points — always follow the specific component manufacturer's instructions. © Adventure Works.